[image: image1.jpg]


[image: image2.wmf] 

[image: image3.wmf] 

 
ConstructionCam Lite Wireless

This 900 MHz non-line-of-site camera system is ideal for locations where bandwidth is 

available (within 30 miles). Best balance of performance and price.

[image: image4.jpg]



[image: image5.wmf] 


Vdsvdsd



EarthCam ConstructionCam Lite Wireless Specification   
         A&E Short Form*

1. Physical

Dimensions: 14.7” H x 8.2” W x 14.5” D (37.4cm H x 20.8xm W x 36.9cm D)

Weight: 11 lbs (4.99 kg)

2. Environmental

Operation Temperature: -20°F to + 120°F (-28.8°C to + 48.8°C)
Thermostatically Controlled Heater 

3. Construction

Engineered Plastic Upper Dome with Aluminum Wall Mounting Bracket and High-impact UV Protected Polycarbonate Lower Dome

4. Camera 

Imager: 1/4" CCD, Sensitivity: 0.1Lux at F/1.6
Video Compression: Motion JPEG and MPEG4, Frames Per Second: 30fps at 4CIF

Resolution: 704 x 576 pixels

Image settings: Day/Night, White Balance, Back Light Compensation, Manual IR-Cut Filter
Lens: F/1.6 – F/4.0, Zoom: 3.5mm – 91mm, 26x Optical 12x Digital

Auto Focus, Manual Day/Night Function

Pan Tilt: Pan Range: +170º to -170º, Tilt Range: -10º to -90º

Communication: 10base-T/100base-TX Ethernet, Public and Private IP Modes for Local IP Assignment
Designed For "EarthCam Control Center - Streaming Edition - Robotic with Mobile Support" Software

5. Communication

RF transmission rate: 1.5Mb/s

Radio channels: 12 non-overlapping

Frequency select: Automatic and manual 
6. Electrical

Operational Voltage: 120VAC (230VAC Available), Power Consumption: 82W

7. Interface and Online Software Features

Software Delivered by Vendor as a Managed Service

Displays Company Logo and Project Name

Capable of Viewing Actual Live Video
Robotic Pan, Tilt, Zoom Control of Camera System
Picture in Picture to View Live Video, While Viewing High Definition Images

Calendar Based Navigation System for Selecting Specific Images

Multifunction Image Browsing 

Pan, Tilt and Zoom Capability within a High Definition Image

Onscreen Control Button for Wiper Control to Allow Remote Cleaning of the Viewing Window

A Multiview Screen to View All Cameras on Project at the Same Time

Graphical Mark-Up Tools for Detailing and Creating Overlays on Images

Graphical Weather Applet Displaying Ten Points of Local Weather Data and 48-hour Forecast 

Remote Solar Monitoring Screen Displaying the DC Amperage Output of Solar Panels

Remote Battery Monitoring Screen Displaying Battery Voltage, Temperature and Status

Remote Cellular Monitoring Screen Displaying Connectivity, Network Traffic and Modem Temperature

Remote Wireless Radio Monitoring Screen Displaying Connectivity, Network Traffic and 

Google Map Features Including Wireless Radio Locations

Share Image Tools: Save, Print, Email and Post to Message board or Mobile Devices

Automated Progress Report in PowerPoint, Open Office and PDF Formats

Map, Aerial and Satellite View by Google

Time-Lapse Feature Includes – Instant Time-Lapse Play Back by Day, Week, Month or Year

Machine to Machine Self-Healing Technology Automates Maintenance of Camera Up to 288 Times Daily

Account Security Features Include – Four Levels of Password Protection, IP Address Block/ Permission and SSL Protection of User Login Password

All Images are the Copyright of the Client and Protected on Secure Servers Owned and Operated by the System Vendor
Product Features





Standard network camera with dome technology and wireless connectivity. Ideal for short term projects.





· All weather dome housing with heater and fan


· Colorful and crisp streaming video


· Pan angle of 340 degrees


· 26x optical, 12x digital zoom 


· 1 lux minimum illumination


· Point-to-point non-line-of-sight wireless connectivity


· Up to 1 mile connectivity (optional antenna upgrade to 30 miles)


· No FCC license required


· 128-bit encryption for data transfer security


· Matched transmitter and receiver


· Will not interfere with Wi-Fi networks


· Plug-and-play, just supply power and start viewing








�





� EMBED Word.Picture.8  ���





� EMBED Word.Picture.8  ���











�





Architectural & Engineering Specifications


    Product:  ECCS05102


   Revised:  06-01-2010





DOWNLOAD ELECTRONIC VERSION OF THIS SPECS





� HYPERLINK "http://www.earthcam.net/aespecs/cclitewireless" ��http://www.earthcam.net/aespecs/cclitewireless�








For additional EarthCam A&E specs visit: � HYPERLINK "http://www.earthcam.net/aespecs/systems" ��http://www.earthcam.net/aespecs/systems�








*Visit EarthCam.net for the full version of this A&E specification.                                6/01 ©2010 EarthCam, Inc.
84 Kennedy Street,  Hackensack,   NJ  07601  USA      Ph.  201.488.1111     Fax.  201.488.1119     info@earthcam.net

_1301836141.doc
[image: image1.jpg]Q EarthCam

== Webcam Technology







_1326549647.doc
[image: image1.jpg]






