EarthCam Webcam Technology                                                                                                    MediaCam
A&E Specifications                                                                                                                Robotic System


EARTHCAM MEDIACAM

PART 2 -  PRODUCT 

2.01 MANUFACTURER

A. EarthCam, Inc.

800.EarthCam (327.8422) toll free

201.488.1111 voice

201.488.1119 fax

84 Kennedy Street,

Hackensack, NJ 07601.

www.EarthCam.net[image: image1.jpg]e

EarthCam




2.02 SYSTEM REQUIREMENTS

A. The outdoor video camera system shall consist rugged camera and heavy-duty construction grade Pan/Tilt, to be mounted as a fixed pole or wall, parapet or non-parapet roof mount.

B. The camera system shall include a Video Webcaster built into an All Weather Enclosure.

C. The camera shall have the ability to take digital still images every 5 or 60 minutes, depending on service package, as well as provide live video.

D. The camera shall upload the still images over a wireless cellular modem, a wireless point-to-point system or a RJ-45 hardware connection to a DSL or cable modem.

E. The content shall be sent to a secure, password protected website with an Interface and Online Software features provided by the Vendors as a Managed Service.

F. The system will operate on 120VAC (230VAC Available) and have a maximum power consumption of 50W.

2.03 EQUIPMENT

A. Heavy Duty Robotic Pan/Tilt Video Camera: Integrated camera and lens assembly consisting of a charge coupled device (CCD) camera with a remotely controlled focal length lens mounted as a permanent module. 

1. Imager: 1/4” 3-CCD.

2. Pixel count: 410,000 pixels X3 NTSC, 470,000 pixels X3 PAL.

3. TV system: both NTSC and PAL video.

4. Sensitivity: .75 lux @ 1/8 sec slow shutter

5. Lens: F/1.6 – F/2.9, zoom: 4.2mm – 84mm, 20x optical - 5x digital

6. Pan range: 340º.

7. Tilt rage: +30º to –53º.

8. Operation speed: panning 50 degrees/Sec, tilting 40 degrees/Sec.

9. Repeatability: +/- 10degrees.

10. Motorized zoom lens: 4.2mm – 84mm.

11. Zoom: 20x optical, 5x digital.

12. Anti Frost: Double sealed glass.

13. Electronic image stabilization.

14. 5°F to 104°F (-15°C to 40°C).

15. Thermostatically controlled heater and defroster.

16. Remotely controllable windshield wiper.

17. Dimensions: 10.7”H x 13.0”W x 13.0”D (329mm H x 272.5mm W x 330mm D).

18. Weight: 22 lbs (10 kg).

B. All Weather Enclosure: Constructed of molded fiberglass polyester Nema 4x with integrated Video Webcaster. Corrosion resistant stainless steel hardware, back plate and locking latches. Brass insert molded and mounting tabs.

1. Video Webcaster:  Single channel video encoder compatible with EarthCam systems.

a. Multi User Level Password Protection for Restricted Camera Access.

b. Events Triggered by Motion Detection, External Inputs or Schedule. 

c. Alarm Notification: E-mail, TCP, and HTTP.

d. Compatible with most versions of Internet Explorer, Firefox and Safari.

e. Video Compression: Motion JPEG, MPEG.

f. Maximum Resolution: 768 x 576.

g. Frames Per Second: 30.

h. IP Addressing: Dynamic or Static.

i. Communication: DSL, Cable Modem, T-1, Wireless ISP, EV-DO Rev-A or LAN Available.

2. Dimensions 14.55" H x12.55" W x8.31" D (37cm H x31.9cm W x21.1cm D).

3. Cable entry: UL rated compression glands.

4. Operational voltage: 120VAC (230VAC Available).

5. Designed for the “EarthCam Control Center - Streaming Video Edition - Robotic with Mobile Support” software.

C. Data Connection: Provide one of the following:

1. In areas with cellular coverage, operate cameras via built-in cellular data connection provided and maintained by the system vendor.

2. In areas without cellular coverage, operate cameras via an RJ-45 Ethernet data connection over broadband or satellite Internet access provided and maintained by the Contractor.

D. Quantity of Cameras: As required by owner.

2.04 INTERFACE AND ONLINE SOFTWARE

A. Remote Access: Contractor’s System Vendor shall provide an online interface system to allow viewing of all digital still images captured and stored during construction, from any location with internet access and with password protection.

1. Maintain images on the System Vendor’s website for reference available at all times during construction and for not less than 60 days after Final Completion.

B. Online Interface Features:

1. Software delivered by vendor as a managed service.

2. Displays company logo and project name.

3. Capable of viewing live video.

4. Robotic Pan, Tilt, Zoom control of camera system.

5. Calendar based navigation system for selecting specific images.

6. Multifunction images browsing.

7. Pan, tilt and zoom capability within a high definition image.

8. Onscreen control button for wiper control to allow remote cleaning of the viewing window.

9. A multiview screen to view all cameras on project at the same time.

10. Graphical mark-up tools for detailing and creating overlays on images.

11. Graphical weather applet displaying ten points of local weather data and 48-hour forecast.

12. Remote solar monitoring screen displaying the DC amperage output of solar panels.

13. Remote battery monitoring screen displaying battery voltage, temperature and status.

14. Remote cellular monitoring screen displaying connectivity, network traffic and modem temperature.

15. Remote wireless radio monitoring screen displaying connectivity, network traffic and Google Map features including wireless radio locations

16. Share images tools: save, print, email and post to message board or mobile devices.

17. Automated progress report in power point, open office and PDF formats.

18. Map, aerial and satellite view by Google.

19. Time-lapse features include – Instant time lapse play back by day, week, month or year.

20. Machine to machine self-healing technology automates maintenance of camera up to 288 times daily.

21. Account security features include – Four levels of password protection, IP address block/permission and SSL protection of User Login Password.

22. All Images are the Copyright of the Client and Protected on Secure Servers Owned and Operated by the System Vendor.

PART 3 -  EXECUTION 

3.01 PREPARATION

A. Unpack camera system neatly. Save packing materials for future shipment of camera system and accessories to Owner or Manufacturer as needed.

3.02 INSTALLATION

A. General:

1. Install camera system following with manufacturer's included instructions, Observe all State and Municipal code and regulations.

2. Install cameras precisely making sure they are plumb, level and at the recommended angle for maximum field of view of operations.

3. Anchor product in place securely making certain there is no movement.

4. Connect cameras to power.

B. Position camera to cover site field of view at approximately 51º horizontal and 39º.

1. Install camera at an elevation for maximum, unobstructed view of site.

2. Make certain the camera location and position prevents any direct light source lens contact for both sunlight and electrically powered light in the field of view through the entire construction period.

3.03 FIELD QUALITY CONTROL

A. Preinstallation Testing: Test camera at ground level on-site before mounting unit in its recommended position.

1. Contact System Vendor 24 hours in advance of installation for test.

2. Connect unit.

3. Contact System Vendor after 30 minutes for remote confirmation that the camera is fully operational.

4. Complete camera installation as designed in plan.

3.04 CLEANING

A. Follow manufacturer’s written recommendations for cleaning cameras.

B. Be sure to clean lenses and all components, including camera-housing windows, and monitor screens or contact System Vendor for professional on-site cleaning and maintenance check.

3.05 INSTRUCTION

A. Contact EarthCam directly by phone to instruct Contractors personnel for instructions on approved methods of adjusting camera equipment.

1. Instruct personnel on all aspects of maintaining equipment and troubleshooting procedures of determining optimum alignment and adjustment of components.

3.06 OPERATION, TERMINATION, AND REMOVAL

A. Maintenance: Camera equipment maintained in good operating condition on a 24/7 throughout contract until removal.

B. Termination and Removal: Camera system removed at the request of the request of the owner when requested. 

1. Camera system including physical accessories and mounting equipment remain the Owners property.

END OF SECTION


© 2010 EarthCam, Inc
 Page 2 of 3                                         
06.16.10


