
[image: image1.jpg]


[image: image2.wmf] 

[image: image3.png]


 
MediaCam

The best EarthCam robotic webcam - a broadcast quality, 3-chip network camera system. 
Superior optics enable remote monitoring of live events, traffic, and weather conditions.
[image: image4.jpg]


[image: image5.wmf] 



    
EarthCam MediaCam Specification   


                      A&E Short Form*

1. Physical
Dimensions: 10.7”H x 13.0”W x 13.0”D 329mm H x 272.5mm W x 330mm D

Weight: 22 lbs (10 kg)

2. Environmental
Operating Temperature: 5ºF to +104ºF (- 15ºC to + 40ºC)

Thermostatically Controlled Heater and Defroster

3. Construction
Heavy-duty rugged outdoor network camera with Pan/Tilt, Windshield Wiper and Ant-Frost glass.  

Pan Range: 340º, Tilt Range: +30º to –53º

4. Camera 
Imager: 1/4" Progressive CCD

Sensitivity: .75 Lux @ 1/8 Sec Slow Shutter

Pixel Count: 410,000 pixels X3 NTSC and 470,000 pixels X3 PAL 

Lens: F/1.6 – F/2.9, Zoom: 4.2mm – 84mm, 20x Optical - 5x Digital

Auto Focus, Remotely Controllable Windshield Wiper

Electronic Image Stabilization, Repeatability: +/- 10degrees

Thermostatically Controlled Heater and Defroster
5. EarthCam Webcaster

Single Channel Video Server Webcaster 5001, Frames Per Second: 30fps, Compression: Motion JPEG, MPEG

Events Triggered by Motion Detection, External Inputs or Schedule and Alarm Notification Via E-mail, TCP, HTTP

Communication: 10Base-T/100Base-TX Ethernet, IP Addressing: Dynamic or Static

Designed for the “EarthCam Control Center - Streaming Video Edition - Robotic with Mobile Support” software
6. Electrical

Operational Voltage: 120VAC (230VAC Available), Power Consumption: 50W

7. Interface and Online Software Features

Software Delivered by Vendor as a Managed Service

Displays Company Logo and Project Name

Capable of Viewing Actual Live Video

Robotic Pan, Tilt, Zoom Control of Camera System

Calendar Based Navigation System for Selecting Specific Images

Multifunction Image Browsing 

Pan, Tilt and Zoom Capability within a High Definition Image

Onscreen Control Button for Wiper Control to Allow Remote Cleaning of the Viewing Window

A Multiview Screen to View All Cameras on Project at the Same Time

Graphical Mark-Up Tools for Detailing and Creating Overlays on Images

Graphical Weather Applet Displaying Ten Points of Local Weather Data and 48-hour Forecast 

Remote Solar Monitoring Screen Displaying the DC Amperage Output of Solar Panels

Remote Battery Monitoring Screen Displaying Battery Voltage, Temperature and Status

Remote Cellular Monitoring Screen Displaying Connectivity, Network Traffic and Modem Temperature

Remote Wireless Radio Monitoring Screen Displaying Connectivity, Network Traffic and 

Google Map Features Including Wireless Radio Locations

Share Image Tools: Save, Print, Email and Post to Message board or Mobile Devices

Automated Progress Report in PowerPoint, Open Office and PDF Formats

Map, Aerial and Satellite View by Google

Time-Lapse Feature Includes – Instant Time-Lapse Play Back by Day, Week, Month or Year

Machine to Machine Self-Healing Technology Automates Maintenance of Camera Up to 288 Times Daily

Account Security Features Include – Four Levels of Password Protection, IP Address Block/ Permission and SSL Protection of User Login Password

All Images are the Copyright of the Client and Protected on Secure Servers Owned and Operated by the System Vendor

Product Features





Deliver cost-effective live video to your television station for on-air applications or public website display.





· Heavy-duty outdoor housing with wiper


· High resolution, 3-chip video and network camera


· 20x optical, 5x digital zoom lens


· Robotic Pan/Tilt/Zoom


· Broadcasts 30fps NTSC video and streaming video


· MPEG capabilities


· Fast, dependable, solid state Linux system


· Wireless and solar options available


· Lease or purchase





�





� EMBED Word.Picture.8  ���





�











�





Architectural & Engineering Specifications


    Product:  ECCS06100


Revised:  06-16-2010





DOWNLOAD ELECTRONIC VERSION OF THIS SPECS





� HYPERLINK "http://www.earthcam.net/aespecs/mediacam" �http://www.earthcam.net/aespecs/mediacam�





For additional EarthCam A&E specs visit: � HYPERLINK "http://www.earthcam.net/aespecs/systems" ��http://www.earthcam.net/aespecs/systems�








*Visit EarthCam.net for the full version of this A&E specification.                        6/16 ©2010 EarthCam, Inc
84 Kennedy Street,  Hackensack,   NJ  07601  USA      Ph.  201.488.1111     Fax.  201.488.1119     info@earthcam.net

_1310282618.doc
[image: image1.jpg]Q EarthCam

== Webcam Technology







