[image: image1.jpg]


[image: image2.wmf] 

[image: image3.jpg]Q ‘/1'7 ¥

P



 
MegapixelCam Mobile

Perfect solution for IP network camera applications where no bandwidth is available. 

This mobile system features an Ethernet interface to high-speed cellular data networks.

[image: image4.jpg]



[image: image5.wmf] 




       
EarthCam MegapixelCam All Weather Mobile Specification  
          A&E Short Form*

1. Physical

Dimensions: 6.87” W x 5” H x 21.2” L (17.45cm W x 12.70cm H x 53.84cm L)
Weight: 14 lbs (6.35 kg)

2. Environmental

Operational Temperature: -10° to 120°F (-23° to 49°C)

Thermostatically Controlled Heater and Blower

24/7 Operation in All Weather Conditions

3. Construction

Powder Coated Aluminum Housing with Stainless Steel Fittings for Padlocks

Impact Resistant Viewing Window, 0.25” (6.35mm) Thick

Two UL Rated Compression Glands, Gas Spring Lid, Adjustable Camera Sled 

4. Camera 

Imager: 6.0 Megapixel, 22.2mm x 14.8mm CMOS 

Lens: F-Stop: F/3.5 – F/5.6, 18mm – 55mm, 3x Optical

Resolutions: 3088 x 2056 Pixels = 6 Megapixels

Auto Features:  ISO, Shutter, White Balance and Focus

Compression: Motion JPEG

Communication: 10base-T/100base-TX Ethernet, IP Addressing: Dynamic or Static

Designed For “EarthCam Control Center - Megapixel Edition – Fixed with Mobile Support” Software

5. Communications

Wireless Cellular Modem EV–DO Rev. A Full Duplex Transceiver with Dual-Band Support for Both 800 MHz Cellular and 1.9 GHz PCS Bands. Forward Link Up to 3.1 Mbps; Reverse Link 1.8 Mbps

6. Electrical

Operational Voltage: 120VAC (230VAC Available). Power Consumption: 68W

7. Interface and Online Software Features

Software Delivered by Vendor as a Managed Service

Displays Company Logo and Project Name

Capable of Viewing Actual Live Video

Calendar Based Navigation System for Selecting Specific Images

Multifunction Image Browsing 

Pan, Tilt and Zoom Capability within a High Definition Image

A Multiview Screen to View All Cameras on Project at the Same Time

Graphical Mark-Up Tools for Detailing and Creating Overlays on Images

Graphical Weather Applet Displaying Ten Points of Local Weather Data and 48-hour Forecast 

Remote Solar Monitoring Screen Displaying the DC Amperage Output of Solar Panels

Remote Battery Monitoring Screen Displaying Battery Voltage, Temperature and Status

Remote Cellular Monitoring Screen Displaying Connectivity, Network Traffic and Modem Temperature

Remote Wireless Radio Monitoring Screen Displaying Connectivity, Network Traffic and 

Google Map Features Including Wireless Radio Locations

Share Image Tools: Save, Print, Email and Post to Message board or Mobile Devices

Automated Progress Report in PowerPoint, Open Office and PDF Formats

Map, Aerial and Satellite View by Google

Time-Lapse Feature Includes – Instant Time-Lapse Play Back by Day, Week, Month or Year

Machine to Machine Self-Healing Technology Automates Maintenance of Camera Up to 288 Times Daily

Account Security Features Include – Four Levels of Password Protection, IP Address Block/ Permission and SSL Protection of User Login Password

All Images are the Copyright of the Client and Protected on Secure Servers Owned and Operated by the System Vendor
Product Features





Enclosed in a heavy-duty, temperature controlled housing, the


MegapixelCam Mobile is a proven reliable outdoor solution for


remote locations.





· 6 megapixel fixed position camera


· Large high definition images: 2,816 x 2,112


· Digital presets and Pan/Tilt/Zoom within captured image


· 10x optical, 5x digital zoom


· Heavy-duty enclosure with heater and fan


· Provides high-speed cellular EVDO and CDMA connection


· Dual-band support: 800 MHz cellular and 1.9 GHz PCS


· High speed data transfer


· Low power consumption


· DHCP and static IP assignment


· Perfect for printing, archiving, and presentations


· Completely plug-and-play, just supply power








�





� EMBED Word.Picture.8  ���





�











�





Architectural & Engineering Specifications


Product:  ECCS03108


Revised:  06-07-2010





DOWNLOAD ELECTRONIC VERSION OF THIS SPECS





� HYPERLINK "http://www.earthcam.net/aespecs/trafficdatasensor" ��http://www.earthcam.net/aespecs/mpmobile�





For additional EarthCam A&E specs visit: � HYPERLINK "http://www.earthcam.net/aespecs/systems" ��http://www.earthcam.net/aespecs/systems�








*Visit EarthCam.net for the full version of this A&E specification.                       6/07 ©2010 EarthCam, Inc.
84 Kennedy Street,  Hackensack,   NJ  07601  USA      Ph.  201.488.1111     Fax.  201.488.1119     info@earthcam.net

_1310282618.doc
[image: image1.jpg]Q EarthCam

== Webcam Technology







