[image: image1.jpg]


[image: image2.wmf] 

[image: image3.png]


 
TrailerCam – MegapixelCam Advanced
Our top-of-the-line megapixel camera featuring live streaming video with wiper system, and 
superior HD images for time-lapse photography.
[image: image4.jpg]



[image: image5.wmf] 





EarthCam Mobile TrailerCam MegapixelCam Advanced Specification      A&E Short Form*
1. Physical

Dimensions: 14.73’ L x 7.5’ W x 8.4’ H (4.49m L x 2.31m W x 2.56m H)

Weight: 2800lbs (1270kg)

2. Construction
Single Torsion-type Axle Steel Welded Trailer with 2” Adjustable Ball Hitch Manual Outriggers 

DOT Approved Lighting Package to Include Brake and Marker Lights 

Trailer Mounted Locking Manual/Motorized 42-Foot Mast with Spiral Conduit for Cable Deployment
Full Size Spare Tire
Guy Wires on Extended Mast to Reinforce and Steady Camera Shot
3. Battery Bank
Charge Controller with Remote Monitoring

Battery Type: Twelve 6Volt DC Deep Cycle Batteries

45 Amp AC Battery Charge

4. Solar Array

500 Watt Solar Array Made of Single Crystal (monocrystalline) Silicon Photovoltaic Modules

5. Camera 
Built-in an Aluminum and epoxy powder painted weatherproof standard IP66/IP67

Enclosure constructed from extruded aluminum and die-cast aluminum end-cover plates

Integrated 16 Megapixel high-definition camera and lens assembly consisting of a metaloxide semiconductor (CMOS) Image sensor camera with  the following features

Imager: 23.6 x 15.6mm CMOS

Resolution: 4928 x 3264  = 16.2 Megapixels

Lens: F/3.5 –F/5.6, 18mm – 55mm

Motion JPEG Compression

Dynamic or Static IP Addressing

Auto Features such as Focus, ISO, shutter, and white balance

Camera Enclosure Dimensions: 6.9”W x 6.6”H x 19.4”L (175mmx68mmx493mm)

Operational Temperature - 10°F to 120°F (- 23°C to + 49°C)

Weight: 16 lbs (7.26 kg)

Designed for “EarthCam Control Center - Megapixel Edition – Fixed with Mobile Support” Software

6. Communication

Wireless GPS Modem EV–DO Rev. A Full Duplex Transceiver with Dual-Band Support for Both 800 MHz Cellular and 

1.9 GHz PCS Bands. Forward Link Up to 3.1 Mbps; Reverse Link 1.8 Mbps and Real Time GPS Reporting

7. Interface and Online Software Features

Software Delivered by Vendor as a Managed Service

Displays Company Logo and Project Name

Capable of Viewing Actual Live Video

Picture in Picture to View Live Video, While Viewing High Definition Images

Calendar Based Navigation System for Selecting Specific Images

Multifunction Image Browsing 

Pan, Tilt and Zoom Capability within a High Definition Image

Onscreen Control Button for Wiper Control to Allow Remote Cleaning of the Viewing Window

A Multiview Screen to View All Cameras on Project at the Same Time

Graphical Mark-Up Tools for Detailing and Creating Overlays on Images

Graphical Weather Applet Displaying Ten Points of Local Weather Data and 48-hour Forecast 

Remote Solar Monitoring Screen Displaying the DC Amperage Output of Solar Panels

Remote Battery Monitoring Screen Displaying Battery Voltage, Temperature and Status

Remote Cellular Monitoring Screen Displaying Connectivity, Network Traffic and Modem Temperature

Share Image Tools: Save, Print, Email and Post to Message board or Mobile Devices

Automated Progress Report in PowerPoint, Open Office and PDF Formats

Map, Aerial and Satellite View by Google

Time-Lapse Feature Includes – Instant Time-Lapse Play Back by Day, Week, Month or Year

Machine to Machine Self-Healing Technology Automates Maintenance of Camera Up to 288 Times Daily

Account Security Features Include – Four Levels of Password Protection, IP Address Block/ Permission and SSL Protection of User Login Password

All Images are the Copyright of the Client and Protected on Secure Servers Owned and Operated by the System Vendor

Product Features





Mobile trailer platform for HD documentation.





· Standalone wireless and self powered system is easy to 


· 16 MP fixed position camera, High Def images: 4928 x 3264 pixels


· Digital presets and Pan/Tilt/Zoom within captured image


· GPS 3G/4G modem 


· Telescoping up to 42' steel mast with an electric extension 


· 130 Watt solar panels 


· AGM sealed battery, 75 Amp battery charger 


· Full size spare tire and removable tongue 


· Guy Wires to stabilize an extended mast and steady the camera shot 


· Real-time remote data management 


· Fast, dependable, solid state Linux system 


· Custom branding to display your company logo


· Lease or purchase





�





� EMBED Word.Picture.8  ���





�











�





Architectural & Engineering Specifications


Product:  ECCS09103


Revised:  05-24-2012





DOWNLOAD ELECTRONIC VERSION OF THIS SPECS





� HYPERLINK "http://www.earthcam.net/aespecs/trailercammpa" �http://www.earthcam.net/aespecs/trailercammpa�





For additional EarthCam A&E specs visit: � HYPERLINK "http://www.earthcam.net/aespecs/systems" ��http://www.earthcam.net/aespecs/systems� 








*Visit EarthCam.net for the full version of this A&E specification.                                 05/24 ©2012 EarthCam, Inc.
84 Kennedy Street,  Hackensack,   NJ  07601  USA      Ph.  201.488.1111     Fax.  201.488.1119     info@earthcam.net

_1310282618.doc
[image: image1.jpg]Q EarthCam

== Webcam Technology







