EarthCam Webcam Technology                                                                                           Silent Inspector 
A&E Specifications                                                                                                           Monitoring System


EARTHCAM SILENT INSPECTOR MONITORING SYSTEM
PART 2 -  PRODUCT 

2.01 MANUFACTURER

A. EarthCam, Inc.

800.EarthCam (327.8422) toll free

201.488.1111 voice

201.488.1119 fax

84 Kennedy Street,

Hackensack, NJ 07601.

www.EarthCam.net[image: image1.jpg]e

EarthCam




2.02 GENERAL DESCRIPTION

A. Silent Inspector is a GPS Monitoring and Data Archiving System. Perfect for dredge and scow management.

2.03 SYSTEM REQUIREMENT

A. The Silent Inspector shall have the ability send GPS data every 5 minutes.

B. The Data shall be sent over a wireless GPS cellular modem.

C. The content shall be sent to a secure, password protected website with an Interface and Online Software features provided by the vendors as a Managed Service.

D. The system will operate on 12VDC and have a maximum power consumption of 25W.

2.04 EQUIPMENT

A. All Weather Electronics Enclosure: Constructed of fiberglass polyester NEMA 4x with integrated Data Encoder. Corrosion resistant stainless steel hardware, back plate and locking latches. Brass insert molded and mounting tabs.

1. Communication: Wireless GPS Modem EV-DO Rev. A full duplex transceiver with Dual-Band support for both 800 MHz cellular and 1.9 GHz PCS bands. Forward link up to 3.1 Mbps, receive link 1.8 Mbps and real time GPS reporting.

2. Data Encoder/Server: Data Encoder will provide real-time vessel data collection with local storage capabilities Integration with ship sensors for remote telemetry monitoring and transfer.

3. Solar power optional (System component vary based on location)

4. Operating temperature: -10°F to 120°F

5. Dimensions: 8.5”H x 13”W x 16”D.

6. Weight: 15 lbs (6.8 Kg).

7. Designed for “EarthCam Control Center – Dredging Edition – Silent Inspector.”
2.05 INTERFACE AND ONLINE SOFTWARE

A. Interface and Online Software Features

1. Software delivered by vendor as a managed service.

2. A multiview screen to view all projects at the same time.

3. Graphical weather applet displaying the ten points of local weather data

4. Share image tools: Save, print, email and post to message board or mobile devices.

5. Aerial and satellite photography library.

6. Time- lapse feature Includes – Instant time-lapse play back by day, week, month or year.

7. Machine to machine self-healing technology automates maintenance of camera up to 288 times daily.

8. Account security features include – four levels of password protection, IP address block/permission and SSL protection of user login password.

9. Monitors scow position, heading, speed, trip number and hull status according to US ARMY CORP OF ENGINEERING (USACE) Silent Inspector specification.

10. Shows historical location on map or Satellite image.

11. Allow Creation and Editing of Geo-Fence region for scow without hull sensors by entering Geo-Coordinates.

12. Sends reports to Army Corps. of Engineering at user-definable intervals according to Silent Inspector specification.

13. Provides calendar to view all past data.

14. Provides interface to manage dredging project information, including scow name, boat name, contract date and reporting frequency.

B. Fleet Management Tools:

1. View position on satellite image.

2. Pop-up display of critical data including heading, speed, position, hull status, weather, solar status and wireless status.

PART 3 -  EXECUTION

3.01 INSTALLATION

A. Installation should be performed by qualified service personnel only in accordance with National Electrical code or applicable local codes.

B. Install camera system following with manufacturer’s included instruction manual.

C. Camera system including physical accessories and mounting equipment shall remain the owners property.

END OF SECTION


© 2009 EarthCam, Inc
 Page 1 of 2                                         
09.21.09


