[image: image1.jpg]


[image: image2.wmf] 

[image: image3.jpg]N

i

—



 
Traffic Data Sensor

EarthCam analysis brings traffic management to new levels! The analytic sensors track activity lane by lane with up to 8-lane accuracy.

[image: image4.jpg]



[image: image5.wmf] 


Vdsvdsd



 

EarthCam Traffic Data Sensor Specification    
                                   A&E Short Form*

1. Physical
Dimensions: 12.6” H x 9.0” W x 3.0” D (32cm H x 22.9cm W x 7.6cm D)

Weight: 4.2lbs (1.9kg)

2. Environmental

Operation Temperature: - 40° F to + 165° F (- 40° C to + 73.8° C)
Accurate Performance: Rain up to 4” (10.2cm) Per Hour, Freezing Rain, Snow, Fog, Wind, Dust

3. Construction
Enclosure is a NEMA 250 Lexan EXL Polycarbonate

Resistant to Corrosion, Fungus, Moisture Deterioration and UV Rays

4. Sensor
Operating Frequency: 10.5 to 10.55Ghz (X-Band)

Beam: Vertical 80º at 6dB two Way Beam, Horizontal 10.5º at 6dB two Way Beam

RF Channels: 4

Accuracy Per-Direction: Volume 96%-98% Speed +/-5mph(8kph), Occupancy +/-10%

Accuracy Per-Lane:  Volume 94%-96% Speed +/- 10mph(16kph), Occupancy +/-20%

Classification Accuracy: 85%

Measured Quantities: 3 Bins Classification, Presence Data for 8 Lanes, Per-Lane Data of Volume, Average Speed, Occupancy and Classification Counts

Detectable Area: Up to 8 Lanes at 9 Feet to 200 Feet (2.7m to 61m)

Communication: Full-Duplex RS-232, Half-Duplex RS-485, Baud Rates From 9600bps to 115200bps

Designed For "EarthCam Control Center - Streaming Edition - Robotic with Mobile Support " Software

5. Electrical
Operational Voltage: 9-28VDC, Power Consumption: 7.5Watts Peak

6. Interface and Online Software Features

Software Delivered by Vendor as a Managed Service

Displays Company Log and Project Name

Traffic Visualization Tool Displaying Overview of Current Traffic Conditions, Headway, Volume by Class and Lane, and Occupancy of Lane

Configurable Alerts for Change in Flow, Speed and Density That Can Be Sent Via Email

Detailed Graphs Displaying Density & Volume by Lane, Volume by Lane & Class of Vehicle, Speed and Flow Data for Overall and by Approach and Lane and Overview of Triggered Alerts 

All Graphs Can Be Viewed by Hour, Day, Week, Month or Year 

Capable of Viewing Actual Live Video and Associated Traffic Data

Robotic Pan, Tilt, Zoom Control of Optional Camera System

Multifunction Image Browsing

Overview Map of Traffic Sensor Location

A Multiview Screen to View All Sensors on Project at the Same Time

Graphical Weather Applet Displaying the Ten Points of Local Weather Data and 48-hour Forecast

Map, Aerial and Satellite View by Google

Machine to Machine Self-Healing Technology Automates Maintenance of Camera Up to 288 Times Daily

Account Management Tools Include – Four Levels of Password Protection, IP Address Block/ Permission and SSL Protection of User Login Password

All Images are the Copyright of the Client and Protected on Secure Servers Owned and Operated by the System Vendor

Product Features





Use accurate wireless vehicle count and speed detection with


illustrated data display software to manage highways and produce 


public reports to divert traffic.  





· Vehicle counting, occupancy and speed detection


· Detects and reports up to eight lanes of traffic


· All weather, all condition performance


· 85% classification accuracy


· Real time analysis for fast response


· Information archived for traffic pattern studies


· Accurate digital traffic reporting


· Easy to install and operate








�





� EMBED Word.Picture.8  ���





�











�





Architectural & Engineering Specifications


Product:  ECWSO6194


Revised:  06-10-2010





DOWNLOAD ELECTRONIC VERSION OF THIS SPECS





� HYPERLINK "http://www.earthcam.net/aespecs/cclitemobile" ��http://www.earthcam.net/aespecs/cclitemobile�





For additional EarthCam A&E specs visit: � HYPERLINK "http://www.earthcam.net/aespecs/systems" ��http://www.earthcam.net/aespecs/systems�








*Visit EarthCam.net for the full version of this A&E specification.                                 6/10 ©2010 EarthCam, Inc.
84 Kennedy Street,  Hackensack,   NJ  07601  USA      Ph.  201.488.1111     Fax.  201.488.1119     info@earthcam.net

_1301836141.doc
[image: image1.jpg]Q EarthCam

== Webcam Technology







