[image: image1.jpg]


[image: image2.wmf] 

[image: image3.jpg]


 
 VelcroCam

   Easy, rapid deployment within 10 minutes of arrival. Ideal for short-term emergency and 

  added security.

[image: image4.jpg]


[image: image5.wmf] 





EarthCam VelcroCam                              




   A&E Short Form*

1. Physical

Dimensions: 14.7” H x 8.2” W x 14.5” D (37.4cm H x 20.8xm W x 36.9cm D)

Weight: 11lbs (4.99Kg)

2. Environmental

Operation Temperature: - 20° F to + 120° F (- 29° C to + 50° C)
Thermostatically Controlled Heater and Defroster

3. Construction

Engineered Plastic Upper Dome with Aluminum Wall Mounting Bracket with Two Quick Release Velcro Straps and High-impact UV Protected Polycarbonate Lower Dome

4. Camera 

Imager: 1/4" CCD, Sensitivity: 0.1Lux at F/1.6
Video Compression: Motion JPEG and MPEG4, Frames Per Second: 30fps at 4CIF

Resolution: 640 x 480

Lens: F-Stop: F/1.6 – F/4.0, Motorized Zoom: 3.5mm – 91mm, 26x Optical 12x Digital

Auto Features: Iris, Focus, White Balance, Backlight Compensation, Day/Night Function, and Electronic Image Stabilization

Pan Tilt: Pan Range: +170º to -170º, Tilt Range: -10º to -90º

Designed For "EarthCam Control Center - Streaming Edition - Robotic with Mobile Support" Software

5. Communication

Wireless Modem EV–DO Rev.A Full Duplex Transceiver with Dual-Band Support for Both 800 MHz Cellular and 1.9 GHz PCS Bands. Forward Link Up to 3.1 Mbps; Reverse Link 1.8 Mbps

6. Electrical

Operational Voltage: 120VAC (230VAC Available), Power Consumption: 68Watts Peak

7. Interface and Online Software Features

Software Delivered by Vendor as a Managed Service

Displays Company Logo and Project Name

Capable of Viewing Actual Live Video

Robotic Pan, Tilt, Zoom Control of Camera System

Picture in Picture to View Live Video, While Viewing High Definition Images

Calendar Based Navigation System for Selecting Specific Images

Multifunction Image Browsing 

Pan, Tilt and Zoom Capability within a High Definition Image

Onscreen Control Button for Wiper Control to Allow Remote Cleaning of the Viewing Window

A Multiview Screen to View All Cameras on Project at the Same Time

Graphical Mark-Up Tools for Detailing and Creating Overlays on Images

Graphical Weather Applet Displaying Ten Points of Local Weather Data and 48-hour Forecast 

Remote Wireless Radio Monitoring Screen Displaying Connectivity, Network Traffic and 

Google Map Features Including Wireless Radio Locations

Share Image Tools: Save, Print, Email and Post to Message board or Mobile Devices

Automated Progress Report in PowerPoint, Open Office and PDF Formats

Map, Aerial and Satellite View by Google

Time-Lapse Feature Includes – Instant Time-Lapse Play Back by Day, Week, Month or Year

Machine to Machine Self-Healing Technology Automates Maintenance of Camera Up to 288 Times Daily

Account Security Features Include – Four Levels of Password Protection, IP Address Block/ Permission and SSL Protection of User Login Password

All Images are the Copyright of the Client and Protected on Secure Servers Owned and Operated by the System Vendor
Product Features





The VelcroCam is a streaming video robotic dome network camera system which is installed easily with a heavy duty velcro strap.





· All weather dome housing with heater and fan


· Colorful and crisp streaming video


· Pan angle of 340 degrees


· 26x optical, 12x digital zoom


· 640x480 high resolution streaming video 


· 1 lux minimum illumination


· Built-in infrared night vision


· Two-way audio and video output functionality


· Wireless EVDO high-speed cellular connection


· Fast, dependable, solid state Linux system


· Heavy-duty velcro strap


· Easy, under 10 minute installation








�





� EMBED Word.Picture.8  ���





�











�





Architectural & Engineering Specifications


Product:  ECCS27100


Revised:  06-14-2010





DOWNLOAD ELECTRONIC VERSION OF THIS SPECS





� HYPERLINK "http://www.earthcam.net/aespecs/trafficdatasensor" ��http://www.earthcam.net/aespecs/velcrocam�





For additional EarthCam A&E specs visit: � HYPERLINK "http://www.earthcam.net/aespecs/systems" ��http://www.earthcam.net/aespecs/systems�








*Visit EarthCam.net for the full version of this A&E specification.                                 6/14 ©2010 EarthCam, Inc.
84 Kennedy Street,  Hackensack,   NJ  07601  USA      Ph.  201.488.1111     Fax.  201.488.1119     info@earthcam.net 

_1310282618.doc
[image: image1.jpg]Q EarthCam

== Webcam Technology







